# Evaluation 3: Payload Effects

## 1. Definition / Purpose

Payload effects evaluation studies how the size of MQTT messages affects system performance, specifically throughput, while keeping the sampling frequency constant.

In real IoT systems:

- Small payloads → faster transmission, less processing

- Large payloads → more metadata, higher overhead

This evaluation helps understand the trade-off between data richness and performance.

## 2. Test Conditions

- Sampling frequency: Fixed at 2 Hz (every 0.5 seconds)

- Pipeline:
Pico W → HiveMQ → Node-RED → InfluxDB → Grafana

- Metric observed: Throughput (messages per second)

Two payload types were tested:

| Payload Type | Payload Size |
| --- | --- |
| Small payload | 76 bytes |
| Large payload | 143 bytes |

## 3. Pico W Code Changes (Payload Size Variation)

### A. Small Payload (76 bytes)

Only essential sensor data was sent.

### Pico W – Small Payload Code Section

json_msg = (

'{"id": %d, "temperature": %.2f, "humidity": %.2f}'

% (msg_id, temperature, humidity)

)

Characteristics

- Minimal fields

- No metadata

- Faster serialization and transmission

### B. Large Payload (143 bytes)

Additional metadata was added to enrich the message.

### Pico W – Large Payload Code Section

json_msg = (

'{"id": %d, "ts_pico": %d, "temperature": %.2f, '

'"humidity": %.2f, "pressure": %.2f, '

'"status": "%s", "device": "PicoW", "location": "Lab"}'

% (msg_id, time.ticks_ms(), temperature, humidity, pressure_hpa, status)

)

Added fields

- Timestamp (ts_pico)

- Pressure

- Comfort status

- Device ID

- Location metadata

## 4. Node-RED Function Used (Payload Size + Throughput)

You used the following Node-RED function:

// msg.payload contains the JSON from Pico

let data = JSON.parse(msg.payload);

// Calculate payload size in bytes

let payloadSize = JSON.stringify(data).length;

// Initialize context variables for throughput calculation

let count = context.get("msg_count") || 0;

let startTime = context.get("start_time") || Date.now();

// Increment message count

count += 1;

context.set("msg_count", count);

// Time elapsed in seconds

let elapsed = (Date.now() - startTime) / 1000;

// Calculate throughput every 10 messages or 10 seconds

let throughput = null;

if (count >= 10 || elapsed >= 10) {

throughput = count / elapsed; // messages per second

// Reset counter

context.set("msg_count", 0);

context.set("start_time", Date.now());

}

// Prepare payload for InfluxDB or logging

msg.measurement = "Payload_Test";

msg.payload = {

temperature: parseFloat(data.temperature),

humidity: parseFloat(data.humidity),

pressure: parseFloat(data.pressure),

payloadSize: payloadSize,

throughput: throughput

};

// Optional: include timestamp for latency calculation

msg.ts_node = Date.now();

return msg;

## 5. Test Results

| Payload Type | Payload Size (bytes) | Throughput (msg/s) |
| --- | --- | --- |
| Small payload | 76 | 0.46 |
| Large payload | 143 | 0.49 |

## 6. Output Interpretation

### Small Payload

- Slightly lower throughput (0.46 msg/s)

- Lower processing overhead

- Faster message creation on Pico

- Less network load

### Large Payload

- Slightly higher throughput (0.49 msg/s)

- Payload size increase did not significantly degrade performance

- System handled extra metadata efficiently

## 7. What This Output Tells Us

- Payload size had minimal impact
Doubling payload size did not cause throughput collapse.

- MQTT and cloud pipeline are efficient
TLS-secured MQTT + Node-RED + InfluxDB can handle moderate payload growth.

- Metadata is practical in real deployments
Adding timestamps, device ID, and location is feasible without major performance loss.

## 8. Summary Statement (Perfect for Report / Viva)

“Payload effects were evaluated by comparing small and extended MQTT messages at a constant sampling frequency of 2 Hz. The small payload of 76 bytes achieved a throughput of 0.46 messages per second, while the larger payload of 143 bytes achieved a throughput of 0.49 messages per second. The results indicate that increasing payload size had minimal impact on throughput, demonstrating that the IoT pipeline can efficiently handle enriched sensor data with additional metadata.”