# Evaluation 2: Throughput

## 1. Definition / Purpose

Throughput measures how many messages the IoT system can successfully process per second from the sensor (Pico W) through:

Pico W → MQTT Broker (HiveMQ) → Node-RED → InfluxDB

It shows the system’s capacity and scalability when the sampling frequency changes.

## 2. Test Scenarios

Two different sampling frequencies were used on the Pico W:

| Scenario | Sampling Interval | Sampling Frequency |
| --- | --- | --- |
| Low rate | 2 seconds | 0.5 Hz |
| High rate | 0.5 seconds | 2 Hz |

This allows comparison between low-load and high-load system behavior.

## 3. Pico W Code Changes / Part Used

The Pico W sampling interval was modified using:

time.sleep(SAMPLING_INTERVAL)

Where:

- SAMPLING_INTERVAL = 2 → Low frequency (0.5 Hz)

- SAMPLING_INTERVAL = 0.5 → High frequency (2 Hz)

Each MQTT message includes a unique message ID:

msg_id += 1

This ensures accurate message counting for throughput calculation.

## 4. Node-RED Function Used (Throughput Calculation)

The following Node-RED function was used to calculate throughput:

let data = JSON.parse(msg.payload);

// Count messages

let count = context.get("count") || 0;

count += 1;

context.set("count", count);

// Store start time

let start = context.get("start") || Date.now();

context.set("start", start);

// Time elapsed in seconds

let elapsed = (Date.now() - start) / 1000;

// Messages per second

let throughput = count / elapsed;

msg.measurement = "Throughput-Test";

msg.payload = {

msg_id: data.id,

throughput_mps: throughput,

temperature: data.temperature

};

return msg;

### What this function does

- Counts received MQTT messages using Node-RED context memory

- Tracks elapsed time since the test started

- Computes throughput (messages per second)

- Stores throughput values for analysis and visualization

## 5. Test Results

| Sampling Frequency | Sampling Interval | Measured Throughput (msg/s) |
| --- | --- | --- |
| 2 Hz | 0.5 s | 0.67 msg/s |
| 0.5 Hz | 2 s | 0.50 msg/s |

## 6. Output Interpretation

### Low Sampling Frequency (0.5 Hz)

- Throughput ≈ 0.50 msg/s

- Matches theoretical expectation

- System processes messages smoothly with minimal delay

- Ideal for low-power and stable operation

### High Sampling Frequency (2 Hz)

- Throughput ≈ 0.67 msg/s

- Lower than theoretical 2 msg/s

- Limited by:

  - Network latency (Wi-Fi)

  - TLS-encrypted MQTT

  - Cloud broker processing

  - Node-RED + InfluxDB write speed

Despite higher load, the system remains stable and does not crash.

## 7. What This Output Tells Us

- System scales but has a ceiling
Increasing sampling frequency does not linearly increase throughput due to cloud and network constraints.

- Pipeline is robust
Even at higher rates, messages are still processed reliably.

- Real-world IoT behavior observed
Performance is affected by security, cloud services, and network conditions — exactly as expected in real deployments.

## 8. Summary Statement (Use in Report / Viva)

“Throughput was evaluated by varying the sampling frequency of the Pico W. At a low sampling frequency of 0.5 Hz, the system achieved a throughput of 0.5 messages per second, closely matching the theoretical value. When the sampling frequency was increased to 2 Hz, the throughput increased to 0.67 messages per second but did not reach the theoretical maximum due to network, encryption, and cloud processing overhead. These results demonstrate that the IoT pipeline is stable and scalable, with realistic performance limitations.”