## Evaluation 1: MQTT Performance

### 1. Definition / Purpose

MQTT performance evaluation measures how efficiently the system communicates sensor data from the Raspberry Pi Pico W to the cloud (via Node-RED).

Key parameters:

- Number of messages sent vs received: Verifies reliability of message delivery.

- Lost messages (% loss): Quantifies any messages that were dropped during transmission, showing communication stability.

A reliable IoT pipeline should have minimal message loss and consistent delivery.

### 2. Testing Process

- Configure the Pico W to send sensor readings at a fixed interval (e.g., every 2 seconds).

- Each message includes:

  - id → unique incremental message number

  - ts_pico → timestamp at the moment the message was sent

  - Sensor data (temperature, humidity, pressure, status)

- Deploy a Node-RED function to receive messages.

- Record timestamps at Pico send and Node-RED receive for each message.

- Count the total messages sent and total messages received over a test period (e.g., 8 minutes).

- Compute lost messages (if any) and latency using timestamps.

### 3. Pico W Code Changes / Part Used

The relevant Pico code section for MQTT performance includes:

# --------------------------

# MQTT message with ID + timestamp

# --------------------------

msg_id += 1

ts_pico = time.ticks_ms()

payload_to_send = ('{"id": %d, "ts_pico": %d, "temperature": %.2f, '

'"humidity": %.2f, "pressure": %.2f, "status": "%s"}'

% (msg_id, ts_pico, temperature, humidity, pressure_hpa, status))

mqtt_client.publish(MQTT_TOPIC, payload_to_send)

Key points:

- Added id → tracks each message.

- Added ts_pico → timestamp when the message is generated.

This allows precise tracking of messages and their delivery timing.

### 4. Node-RED Function Used

Your Node-RED function for this evaluation:

let data = JSON.parse(msg.payload);

// Timestamp from Pico

let ts_pico = data.ts_pico;

// Timestamp when Node-RED received message

let ts_nodered = Date.now();

// Store arrival timestamp

msg.payload = {

id: data.id,

ts_pico: ts_pico,

ts_nodered: ts_nodered,

temperature: data.temperature,

humidity: data.humidity,

pressure: data.pressure,

status: data.status

};

return msg;

Explanation of the function:

- Parses JSON from Pico.

- Records Node-RED timestamp (ts_nodered) for latency calculation.

- Passes all data to the next node (InfluxDB or logger) for analysis.

### 5. Test Output

- Number of messages sent: 241

- Number of messages received: 241

This indicates:

- No message loss occurred during the test.

- Reliability of the MQTT communication is 100%.

Other potential outputs (not given here, but possible for analysis):

- Latency per message: ts_nodered - ts_pico

- Average latency over all messages → shows how fast messages travel from Pico to Node-RED.

### 6. What This Output Tells Us

- Communication is stable: All messages sent from Pico W were successfully received by Node-RED.

- MQTT broker is reliable: HiveMQ handled the messages without drops.

- No immediate need for retransmission: Message loss is 0%.

- Foundation for further evaluations: Since all messages were received, we can now confidently evaluate throughput and payload effects without interference from message drops.

✅ Summary Statement (for report/presentation):

"During the MQTT performance test, 241 messages were sent from the Raspberry Pi Pico W, and all 241 messages were successfully received by Node-RED. This indicates 0% message loss, confirming the reliability of our MQTT communication pipeline. Each message included a unique ID and a timestamp, enabling precise tracking and latency analysis."